Set up a Dial Up Networking File

From the Start button select  Programs / Accessories / Communications / Dial-up Networking

Double Click Icon Labeled:  Make New Connection

- For Type a name for the comp you are dialing : Type what describes your service- via Satellite

- For Select a device select : Worldphone data modem (or Hayes Compatible 9600 modem that was setup for your Inmarsat terminal) .  Click next.

- Area code : Type nothing,

- Phone number : type 00 (country code)(area code)(number to service) Followed by #

# Example: 0019545551212#

- Country Code : your choice. Click next.Click finish.

If you can't create this connection, it is possible that you havn't downloaded the dedicated flopy disk relating to the Dial-up Networking . They are given with the basic kit window 95.

Next you must check your Configuration, to do this perform the following :

From Programs/Accessories/Communications/Dial-up Networking,

Right Click the Dial-up Icon you created, scroll down and Select Properties button.

General TAB : Un-check Use area code and dialing properties.

At the bottom click Configure Button.

Under: General Tab

Port : Select com port 1

Maximum speed : select 9600.

Under: Connection tab

Click port settings button and uncheck use FIFO.

Click Ok.

Under: Options Tab – Leave all settings default.

Click Ok.

Server Types TAB :

Under Type of Dial-Up Server :

Make sure: PPP:Windows95, Windows NT 3.5, Internet  is selected.

In the options below CHECK ONLY the following boxes:

### Log onto network

Enable Software compression

TCP/IP

## The Following is necessary when using an internet company as an E-mail provider

In Allowed Network Protocalls:

If you have the DNS entry number for your service provider

click button [TCP/IP settings]

Server assigned IP address must be checked.

Specify name server addresses must be checked.

Under primary DNS type the numbers provided to you by your Internet provider.

Leave all other tabs set for default unless your Data Software requires changes.

Click [OK], Click [OK].