NERA Mini-M QUICK START GUIDE

Thank you for your purchase of your Inmarsat Phone from GMPCS.  GMPCS Personal Communications is pleased to have you as a customer of the Inmarsat Phone system.  Please read this Quick Start Page and your User’s Manual thoroughly before making any phone calls.  By doing this you will ensure proper use and care of the terminal while in your possession.

The Phone you have purchased is the NERA WorldPhone.  It has been commissioned by GMPCS Personal Communications on the global Network of France Telecom utilizing the Inmarsat satellite system.  The phone has been pre-set to the GMPCS network [NET PROVIDER], if this setting is changed, you may be subject to surcharges and excess fees as outlined in the GMPCS Service Agreement.

The phone numbers assigned to your unit are displayed on a sticker under your handset.

USING YOUR MOBILE TERMINAL

- Before using your Mobile Earth Station (MES), you may have to charge your battery.  Charge new batteries for at least 16 hours with the AC to DC power plug to prolong battery life.  Try to drain batteries completely before re-charging to extend battery life.

- Next, you must determine your location and where to point your MES.  There are four satellites to chose from: Atlantic Ocean Region East (AORE), Atlantic Ocean Region West (AORW), Pacific Ocean Region (POR), Indian Ocean Region (IOR).

- Determine your location, position the antenna (azimuth & elevation) in the general position of the satellite, and power up the unit.  The phone will look for the last satellite it used.  If this satellite is not in your area, or if you are pointing to a different one, you will have to change the satellite setting.  PRESS [SEEK] to search for a different satellite.  The unit will initialize with a spot beam of the satellite.  You can adjust the reception by moving the antenna

- IMPORTANT!  Once initialized on the satellite, the display will show the satellite and the Network Service Provider  [GMPCS].  Display should read something like:

“011 GMPCS in AORW  Dial 00 + Country Code + Subscriber number ”

Your terminal has been programmed to work with France Telecom prior to shipment for your protection.

DIALING INSTRUCTIONS

Calling from the MES (Mobile Earth Station) to Land    Dial (“00” + country code + city code + Number + “#”), 
                                                                                               Whereby “00”= direct dial call, and “#” is equivalent to SEND.

Calling from Land to MES (Mobile Earth Station)

**NOTE** : Check with your long distance phone company for the charges to call the MES.  We offer a 2-stage dial service that could greatly reduce your costs calling shore to MES.

- The terminal can be logged onto one of 4 satellites.  Each satellite has its own specific “Code”.  The following are the possibilities for dialing the MES.  International Access Number from the United States is 011

- Atlantic Ocean Region East		(international access number)+(871) + (Mobile number )
Pacific Ocean Region			(international access number)+(872) + (Mobile number )
Indian Ocean Region			(international access number)+(873) + (Mobile number )
Atlantic Ocean Region West		(international access number)+(874) + (Mobile number )

- Some countries support  One Code

For all of the satellites			(international access number)+(870) + (Mobile number )