# Inmarsat Thrane & Thrane M4 Satellite Telephone

Setting up Satellite Telephone				Serial #:  XXXXXXXX											ISN:  76TT71-XXXXXX

·	Refer to the satellite coverage maps to find the satellite that covers your area.

·	Refer to the satellite azimuth and elevation maps to find the general direction in which you will point the antenna and its elevation to reach  the chosen satellite.

·	Locate a suitable stable location for the unit. You must have a clear view to the satellite.

·	Use the enclosed 4 m antenna cable if you want to locate the antenna away from the electronics unit

·	Lift the antenna to the approximate elevation and azimuth.

·

·	Press the Power button and allow the Mini-M to start up

·	The M4 will show “ wait for NCS”

Aligning the antenna

·	On the handset are 4 LED's; the first one to the left (green light on) indicates that the terminal is on, the last one on the right indicates if a signal is received from the satellite (green light on).

·	The antenna has a built-in beeper that will help you locate the satellite and get the strongest possible signal.  Turn the beeper on by pressing the keys 2nd and then  9.  The antenna beep tone will get higher when the signal is stronger.  You can also see the signal strength with the C/No number, try to get this number higher than 50. You can also see the signal strength with the bars on the top center of the display. Weak signal is one or two bars, you should achieve 3 to 5 bars for optimal transmissions.

·	Listening to the beep or watching the bars on the display,  move the antenna slightly left and right until you locate the satellite. When the satellite is located, the satphone will display the number of the spot beam it has selected. You should then fine tune by moving the antenna slightly left and right and then pausing for a few seconds to allow the signal strength indicators to adjust, until you get the strongest signal.

·	Fine tune the elevation by adjusting the antenna support slightly and then pausing for a few seconds to allow the signal strength indicators to adjust. Turn off the beeper by pressing the keys 2nd and then  9. Press the exit button.

Once the signal is received the screen will show the satellite and GMPCS (because this unit is a beta unit, you sometimes have to press the exit button to get back to the initial screen)

Make sure that GMPCS is selected to receive the preferred billing.

If another station is selected please refer to the manual to select the preferred LES.(instructions below)

2nd

menu

Sat Setup, OK

LES, OK

Default LES, OK

for 3 of the 4 satellites (AOR-W, AOR-E & IOR)  choose Station 011 France Telecom and press ok

(it will put you back to the start and you will follow the steps above for each of the 4 satellites)

Dialing Voice, Fax Machine and Data calls from the terminal.

·	To establish a voice call lift the handset, enter number on keypad: 00+Country code+Area code+Phone # (e.g. 00-1-403-640-0550) and press the # key .

- To establish a voice call from an analog phone connect an analog telephone (including STUIII) to the first RJ11 connector of the M4. Lift the handset, enter number on keypad: 00+Country code+Area code+Phone # (e.g. 00-1-403-640-0550) and press the # key .

- To establish a fax connect fax machine to the first or second RJ11 connector of the M4. press “speaker” on your fax machine and enter the fax number on your fax machine keypad: 00+Country code+Area code+Phone+# (e.g. 00-1-403-640-0342-#).  Wait for the connection and press the start button when you hear the sound of the receiving fax machine  or when the satphone display prompts you to press start.

Note: if you are faxing via a PCMCIA modem fax card, just enter the fax number in your computer: 00+Country code+Area code+Phone+# (e.g. 00-1-403-640-0342-#) and choose send.

- To establish a data call enter the data number in your computer: 00+Country code+Area code+Phone+# and choose connect.

Dialing ISDN Data calls from any ISDN devices.

To establish an ISDN data call connect your ISDN device to the RJ45 of the M4 and enter the data number in your computer (or the ISDN device): 00+Country code+Area code+Phone and choose connect.

Receiving an incoming Voice or Fax call.

·	If your terminal is correctly setup it can receive incoming calls.

·	When the unit rings for a voice call, simply lift hand set and speak.

·	This unit will also receive a fax/data automatically if you set your fax machine/computer  on auto answer/ in host mode.

Dialing into the Inmarsat Mini-M terminal

·	To dial to the unit simply dial direct.  The Numbers for your terminal are:

Handset VOICE  Mini-M:			011-87X-762-XXX-XXX

1st RJ-11 High Speed voice (speech): 		011-87X-762-XXX-XXX

ISDN 64k  DATA:					011-87X-762-XXX-XXX

There are other numbers for various services, see attached form.

- The first group of numbers (above shown as “!!”) represents the code for international calls in your area (e.g. 00 in Europe or 011 in North America). I.e. calling from USA to an M4 that is logged on to the AOR-W on STUIII call 011-874-600-XXX-XXX.

- The second group of numbers (above shown as “???”) identifies the satellite.  See the map for the satellite regions.

871 - Atlantic Ocean East Region (Europe)

872 - Pacific Ocean Region

873 - Indian Ocean Region

874 - Atlantic Ocean West Region (Americas)

·	The third group of numbers identifies the specific terminal numbers.

If you have any questions please call GMPCS Personal Communications, at 1-954-973-3100, or via fax at 1-954-973-4800 or via e-mail:    gino@gmpcs-us.com  Gino Jensen: 1-954-973-3100 , ext. 201 , Cell:  954-224-1010